Use Attainability Analyses and Existing Uses

June 16, 2003

Prepared by Indianapolis Clean Stream Team for Public Discussion

The City of Indianapolis is revising its April 2001 Long-Term Control Plan for reducing combined sewer overflows to Marion County streams.  Once completed and approved by the U.S. Environmental Protection Agency and Indiana Department of Environmental Management, the plan will dramatically reduce the frequency and duration of combined sewer overflows and significantly reduce the volume of raw sewage flowing into neighborhood streams and the White River.

The city is evaluating a variety of technologies and CSO control alternatives, including storage tunnels and tanks, on-site treatment facilities, and a combination of storage and on-site treatment.  These CSO control alternatives may be combined with sewer separation in isolated areas, improved stormwater management, conversion of neighborhoods on septic systems to sewers, and stream corridor restoration as the the city adopts an integrated watershed approach to improving water quality.  The plan also will include expansion projects at the Belmont and Southport Advanced Wastewater Treatment Plants to enable the plants to treat more flows during and after wet weather.

A critical question in preparing the long-term control plan is the recommended size of storage tunnels, tanks and on-site treatment facilities.  The larger the facilities, the more sewage and stormwater they will capture and the fewer times overflows will occur.  However, as size increases, so does the cost.  The city, in conjunction with the community, will review the costs and benefits of the various alternatives, and seek consensus behind a plan that will best protect public health and the environment in an affordable and cost-effective way.  Although overflows will be reduced from the current 60 events per year to 12 or fewer events, the city cannot completely eliminate sewer overflows because some storms inevitably will be too large for the storage and treatment facilities.

Under the federal Clean Water Act, a state must prescribe “designated uses” for each water body, and then establish water quality standards (WQS) to protect those uses.   Indiana designated all water bodies for aquatic life and full body contact recreation uses, and established standards to protect those uses.  However, Indiana’s recreational standards for E. coli bacteria cannot be met when there are sewer overflows.  Through the long-term control plan, the city’s goal is to meet Indiana’s water quality standards during dry weather, and to reduce the severity and frequency of violations during wet weather.

Under Indiana law, combined sewer communities can seek a temporary suspension of the designated use and associated water quality standards during wet weather by conducting a use attainability analysis, or UAA.  The City of Indianapolis intends to seek a suspension of the full-body contact recreational use on those stream reaches when and where sewer overflows will still occur.  The proposed suspension will be based upon travel time and computer model predictions of how long it will take streams to recover from a sewer overflow event.  However, it will last no more than four days, as required under Indiana law.  In seeking a suspension of water quality standards, the city is not abandoning the goals of the Clean Water Act.  Rather, we are seeking some degree of legal protection while we determine what level of water quality can be achieved in an urban setting.

The city already has devoted significant resources to reducing combined sewer overflows.  Recently, the city broke ground on a project to install a $1 million inflatable dam in a combined sewer along Pogues Run in Brookside Park.  This inflatable dam, and six others like it, will significantly reduce sewer overflows in priority areas around the city.  Millions of dollars in additional investments are being made at the treatment plants and in other early CSO control projects to reduce sewer overflows now, even before negotiations with IDEM and EPA are complete.

This paper describes the regulatory framework that guides use attainability analyses, including the concept of protecting existing uses.  It represents the Indianapolis Clean Stream Team’s legally defensible interpretation of federal and state law and regulations as they pertain to these issues.  It also identifies several policy concerns and questions that need to be resolved in order for CSO communities to meet the goals of the Clean Water Act and Indiana water pollution control laws.

Federal Regulatory Framework

The Clean Water Act

One of the express goals of the Clean Water Act is that “wherever attainable, an interim goal of water quality which provides for the protection and propagation of fish, shellfish, and wildlife and provides for recreation in and on the water be achieved.”  See CWA Section 101(a)(2), 33 U.S.C § 1251(a)(2).  The Act also requires that when a state adopts or revises water quality standards, those standards must include and protect designated uses such as “public water supplies, propagation of fish and wildlife, recreational purposes, and agricultural, industrials, and other purposes.”  See CWA Section 303(c)(2)(A), 33 U.S.C § 1313(c)(2)(A).

Water Quality Standards Regulations

Federal regulations implementing the above-referenced provisions of the Clean Water Act describe the requirements and procedures for “developing, reviewing, revising, and approving water quality standards” by the states.  40 CFR § 131.1; see generally 40 CFR Part 131.  A state must conduct a UAA whenever the state wishes to remove a designated use that is specified in Section 101(2)(2) of the Clean Water Act.  40 CFR § 131.10(j).  A UAA is “a structured scientific assessment of the factors affecting the attainment of the use which may include physical, chemical, biological, and economic factors as described in Sec. 131.10(g).”  40 CFR 131.3(g).  However, a state may remove a designated use from its water quality standards only if the designated use is not an existing use.  40 CFR 131.10(g) and (h)(1).

“Existing uses” are defined as “those uses actually attained in the water body on or after November 28, 1975, whether or not they are included in the water quality standards.”   40 CFR 131.3(e). This federal regulation does not specify how to determine whether a use has been “actually attained.”

Pursuant to these federal regulations, the UAA provides the scientific, technical and/or economic support for a state’s determination that a designated use is not attainable based on one or more of the six factors listed in 40 CFR 131.10(g). The six factors are:

- Naturally occurring pollutant concentrations may prevent the attainment of the use;

- Natural, ephemeral, intermittent, or low-flow conditions or water levels may prevent the attainment of the use, unless these conditions may be compensated for by the discharge of sufficient volume of effluent discharges without violating state water conservation requirements to enable uses to be met;

- Human-caused conditions or sources of pollution may prevent the attainment of the use and cannot be remedied or would cause more environmental damage to correct than to leave in place;

- Dams, diversions, or other types of hydrological modifications will preclude the attainment of the use, and whether it is feasible to restore the water body to its original condition or to operate such modification in a way that would result in the attainment of the use;

- Physical conditions related to the natural features of the water body, such as lack of a proper substrate, cover, flow, depth, pools, riffles, and the like, unrelated to (chemical) water quality, that preclude attainment of aquatic life protection uses; or

- Controls more stringent than those required by sections 301(b) and 306 of the Clean Water Act would result in substantial and widespread economic and social impact.

A state must hold a public hearing and then submit the UAA to the U.S. Environmental Protection Agency (EPA) for review and approval.  See 40 CFR §§131.20(b), (c) and 131.21.  The public hearing can be accomplished in conjunction with public comment periods for discharge permit reissuance or revision.

EPA Policy and Guidance Documents

EPA’s Combined Sewer Overflow (CSO) Control Policy (59 Federal Register 18688, April 19, 1994) provides guidance on how the federal legal framework applies to CSO communities.  The CSO Control Policy states that “development of the long-term plan should be coordinated with the review and appropriate revision of water quality standards and implementation procedures on CSO-impacted waters to ensure that the long-term controls will be sufficient to meet water quality standards.”  The Policy notes that states may not remove a designated use from its water quality standards if it is an existing use.  As part of the analysis, “States should evaluate whether the designated use could be attained if CSO controls were implemented.”  (EPA CSO Control Policy at III.B, paragraph 2)

Historically, UAAs have been applied to aquatic life almost exclusively, and nearly all guidance available from EPA deals with aquatic life UAA methodologies.  However, on February 27, 1998, EPA Region 1 approved a UAA that changed recreational water quality standards for  Boston Inner Harbor to “SBCSO”  -- a designation that indicates that the water quality standard is achieved when Massachusetts Water Resources Authority completes the CSO abatement facilities in the approved Final CSO Facilities Plan.  In response to an express congressional directive, EPA also issued a Guidance Document in July 2001 that provides additional guidance with respect to UAAs for recreational uses.  See “Guidance:  Coordinating CSO Long-Term Planning with Water Quality Standards Review” (EPA Office of Water – July 31, 2001).

The July 2001 guidance lays a foundation for integrating water quality standards reviews, implementation of high-priority CSO controls, and development of well-designed and operated LTCPs that support attainment of water quality standards without causing substantial and widespread economic and social impacts.  The guidance states that EPA’s goal “is for CSO communities to develop and implement cost-effective LTCPs that achieve compliance with applicable water quality standards and with other CWA requirements, and for states to review and revise water quality standards as appropriate to ensure they are attainable.”  The guidance also states that if “chemical, physical or economic factors appear to preclude attainment of the use, the data collected during the planning process may be used to support revisions to water quality standards.  These revisions could include adoption of uses that better reflect the water quality that can be achieved with a level of CSO control that does not cause substantial and widespread economic and social impact.”  See July 31, 2001, Guidance, page 2.

State Regulatory Framework

Indiana’s Designated Uses Pursuant to the Clean Water Act

Indiana designated all water bodies for aquatic life uses and full body contact recreation and it adopted standards to protect those uses.  The problem for most Indiana CSO communities, including Indianapolis, is that any discharge from a CSO will cause exceedences of Indiana’s recreational standards for E. coli bacteria. Eliminating all CSO discharges would cause widespread economic and social impact for many Indiana CSO communities, and achieve little, if any, additional actual use given natural conditions, urban stormwater and other discharges that likely would still cause exceedances of the E. coli standards even if CSOs could be fully controlled.

Senate Enrolled Act 431

During its 2000 session, the Indiana General Assembly addressed this issue in Senate Enrolled Act 431.  Section 20(a) of the statute provides that “designated uses and associated water quality criteria are temporarily suspended on a site specific basis, for waters affected by discharges from combined sewer overflow points listed in the National Pollutant discharge Elimination System (NPDES) permit due to wet weather events,” if specific conditions are met, including the federal requirements relating to the UAA process.  See IC 13-18-3-2.5(a).  This statute expressed the will of the legislature and the Governor that Indiana’s CSO control policy would allow for the temporary suspension of designated uses and water quality standards, as allowed under federal law.  Section 24(d) required the Indiana Department of Environmental Management (IDEM) by October 1, 2000, to “provide guidance to all combined sewer overflow communities explaining the requirement of the use attainability analysis and the long term control plan to aid communities in determining how to comply with the requirements.”

IDEM CSO LTCP and UAA Guidance

IDEM issued its final Combined Sewer Overflow Long Term Control Plan and Use Attainability Analysis Guidance (Guidance Document) on September 19, 2001, which became effective on December 14, 2001.  In this Guidance Document, IDEM has included the six factors for conducting a UAA provided in 40 CFR 131.10(g). Since the current recreational E. coli bacteriological criteria is the main focus of concern for most Indiana communities, IDEM’s Guidance Document focuses on the designated recreational use and the bacteriological criteria and is geared toward those CSO communities interested in seeking a temporary suspension of that use.

IDEM’s Guidance Document identifies the steps that must be followed to apply for and obtain a temporary suspension of a designated use:

1.	IDEM must determine if a designated use is an existing use.  Through the UAA process, the CSO community provides the information for IDEM to make this determination (subject to EPA approval).

2.	IDEM must determine whether any of the six factors in 40 CFR 131.10 would prevent the attainment of a designated use. Through the UAA process, the CSO community provides the information for IDEM to make this determination (subject to EPA approval).

3.	The CSO community must submit the UAA and LTCP to IDEM.

4.	IDEM must determine whether the UAA and LTCP are sufficient to justify that the designated use is not attainable.

5.	IDEM must conduct a public hearing.

6.	IDEM must submit the proposed use suspension to EPA for approval.

7.  IDEM must modify the CSO community’s NPDES permit and Water Quality Standards

8.  EPA must approve the proposed use suspension and changes in Water Quality Standards.

The purpose of this paper is to discuss the policy and legal implications of the first step:  determining whether a designated use is an existing use that must be protected under the federal Clean Water Act.  If the designated use is an existing use, then the designated use cannot be removed and the existing use must be protected.  In discussions with stakeholders, we have found that there are two important issues associated with the existing use question:

- How should the term “existing use” be defined?  IDEM’s guidance interprets actual uses of water bodies to equate to existing uses in most circumstances.  If recreation occurs in a water body, an existing use is presumed.  The city, on the other hand, maintains that achieving water quality standards is an essential element of determining an existing use under the law.  Federal law and regulations are not entirely clear on this point, and EPA guidance and internal memoes are sometimes in conflict with each other.

- Second, what are the implications to a community if there is an existing use?  Is affordability still a factor that can be considered in developing a long-term control plan for streams where there are existing uses, or is compliance with water quality standards required, regardless of cost?  If affordability is a factor and yet the use cannot be temporarily suspended, what mechanism will protect a community from legal action over water quality standards violations from remaining sewer overflows?

These questions should be answered in order to provide clear direction to CSO communities as they evaluate alternatives and prepare long-term control plans, with an ultimate goal to protect public health, improve water quality and maintain compliance with regulatory requirements.

What is an Existing Use?

At first blush, the question “What is an Existing Use?” seems straightforward and easy to answer.  Outside of the regulatory context, one might be inclined to equate the question with “Did a use actually occur?”  If yes, then the use is an “existing use.”  But the regulation does not define existing uses as “those uses that actually occurred in a water body on or after November 28, 1975.”  Instead, the regulation uses the language “actually attained.”  When considered in the context of federal water quality standards regulations, there is a distinct difference between “actually occurred” and “actually attained.”

Water Quality Standards Regulations

Section 131.2 of the federal water quality standards regulations explains the purpose of water quality standards.

A water quality standard defines the water quality goals of a water body, or portion thereof, by designating the use or uses to be made of the water and by setting criteria necessary to protect the uses.  States adopt water quality standards to protect the public health or welfare, enhance the quality of water and serve the purposes of the Clean Water Act (the Act).  “Serve the purposes of the Act” (as defined in sections 101(a)(2) and 303(c) of the Act) means that water quality standards should, wherever attainable, provide water quality for the protection and propagation of fish, shellfish and wildlife and for recreation in and on the water and take into consideration their use and value of public water supplies, propagation of fish, shellfish, and wildlife, recreation in an on the water, and agricultural, industrial, and other purposes including navigation.

Such standards serve the dual purposes of establishing the water quality goals for a specific water body and serve as the regulatory basis for the establishment of water-quality-based treatment controls and strategies beyond the technology-based levels of treatment requires by sections 301(b) and 306 of the Act. (40 CFR § 131.2 (emphasis added).)

In other words, uses are designated not because the uses actually are occurring in a water body, but because the uses reflect a water quality goal to be attained for a particular water body consistent with the purposes of the Clean Water Act.  At the time a particular use is designated by a state, that use might not be occurring and the water quality to protect and support that use may not yet be attained.  Protective water quality is attained through the “establishment of water-quality-based treatment controls and strategies” designed to meet the water quality standards criteria for specific pollutants.

The water quality regulations recognize, however, that even after a state designates a particular use for a water body, attaining the water quality to support that use may not be feasible.  Therefore, the regulations allow a state to remove a designated use if the state “can demonstrate that attaining the designated use is not feasible” because of one or more factors outlined in the regulation.  40 CFR § 131.10 (g).  The state makes this demonstration through a use attainability analysis.  40 CFR § 131.10(j)(2).  However, this relief is available only if attaining water quality to protect a use is not feasible.  Once the water quality goal to protect a use, whether designated by a state or not, has been attained, the water quality standards regulations ensure that that level of water quality is maintained, first through an “antidegradation policy,” see 40 CFR § 131.12, and second, through the concept of “existing use,” see 40 CFR § 131.10 (g) and (h).

When considered within this context, the meaning of the term “actually attained” in the existing use definition seems clear.  “Actually attained” refers to the attainment of the water quality necessary to support a particular use, whether or not that use actually has occurred in the water body.  It would be inconsistent with the purposes of the Clean Water Act to allow degradation of water quality once it has been attained on or after November 28, 1975.  Conversely, if the water quality necessary to protect a particular use has not been attained on or after November 28, 1975, then that use is not an existing use.  This interpretation of the existing use regulation is supported by and consistent with relevant EPA guidance documents, federal law and other EPA materials, as shown below.

EPA Guidance Documents and Relevant Materials on Existing Use

Unfortunately, a clear and comprehensive analysis of the recreational existing use issue is not included in EPA’s 1994 Combined Sewer Overflow (CSO) Control Policy or in early EPA Guidance Documents on water quality standards review.  The reason for this may be that most EPA guidance on conducting a UAA has addressed the aquatic life designated use.  If water quality is not sufficient to protect aquatic life, then aquatic life will not be present, and therefore the issue of existing use does not arise.  The recreational designated use poses a different situation.  People may choose to recreate in a water body even if it has not attained the water quality necessary to support the recreational use.

Nevertheless, even though the term “actually attained” is not defined and the process for determining whether a use has actually been attained has not been explicitly specified, EPA guidance documents and other relevant materials support a water quality-based interpretation.  For example, in EPA’s April 27, 1995, Interim Economic Guidance for Water Quality Standards, the agency describes the steps needed to demonstrate substantial and widespread economic and social impacts of attainment of designated uses.  The first step listed in Table 4 of this Guidance is “Demonstrate that designated use is a potential use and not an existing use.”  The only information required from the applicant to meet this step is “Data from State Water Quality Assessment Documents and water quality standards regulations.”  This demonstrates that EPA does not consider a use to be actually attained when the water quality does not support that use.

Likewise, in 1998, EPA issued an Advanced Notice of Proposed Rulemaking seeking comments from interested parties on possible revisions to the Water Quality Standard regulations.  See Advanced Notice of Proposed Rule Making (ANPRM) to amend the national WQS regulations, 63 Fed. Reg. 36741-36806 (July 7, 1998).  In the notice, EPA provided background information on “uses,” and stated that “[d]esignated uses focus on the attainable condition while existing uses focus on the past or present condition.  Section 131.10 then links these two broad use categories in a manner which intends to ensure that States and Tribes designate appropriate water uses, reflecting both the existing and attainable uses of each water body.  For this discussion it is important to consider both the distinction between and the linkage of designated and existing uses.”  63 Fed. Reg. at p. 36748 (emphasis added). It seems clear that the “condition” to which EPA refers is water quality and “existing” implies that the water quality standards are already attained – in contrast to attainable for a designated use.

EPA also provided specific information about making an existing use determination:

In making an existing use determination, there is a link between the use and water quality.  To be considered an existing use, the use must have been actually attained in the past, is now attained or water quality is sufficient to support the use.  However, for some sites, water quality, alone, may be an insufficient basis for making an existing use finding if there are other factors that would prohibit the use from taking place regardless of the quality of the water at a site. (63 Fed. Reg. at p. 36753.)

This makes clear that water quality is the threshold issue in making an existing use determination.

EPA expanded upon this in the context of determining a recreational existing use.  According to EPA, a recreational use is not an existing use when the water body is not suitable for swimming because the water quality and physical characteristics do not support that use.

Obviously, any decision about whether or not a use is an “existing use” must be a water body–specific determination.  The existing use determination is, therefore, site-specific, and decisions should consider water quality and other limiting factors such as the physical habitat specific to a particular water body.  A few examples may help illustrate the issue.  A somewhat common existing use question applies to primary contact recreation: if a few people on a few occasions ‘swim’ in a water body that does not have the quality or physical characteristics to support swimming, is this an existing use, even if the water body is posted ‘no swimming’ due to bacterial contamination and lacks the physical features to actually support swimming?  The straightforward answer to this question is that ‘swimming’ is not an existing use because the present (or past) condition does not support that use.  This conclusion is based on the very limited actual ‘use’ and, more importantly, the lack of suitable water quality and physical characteristics that would support a recreational swimming use now or in the future (as determined by the water quality requirements and recreational swimming considerations, including safety considerations, in the State or Tribal classification system for primary contact recreation).

A question has been raised as to how to interpret the regulation in the context of this example. One could determine that because the water body is not suitable for swimming, and has not been since 1975, primary contact recreation is not an existing use.  Alternatively, one could determine primary contact recreation to be an existing use because the water body was actually used for swimming, even though the use was occasional and water quality and physical characteristics were not acceptable to support such a use.  EPA believes the first alternative is the better interpretation of Agency regulations and guidance in this example, because the use is not established and the water quality and other factors would appear to prohibit actually attaining a recreational swimming use.  63 Fed. Reg. at pp. 36752-53 (emphasis added).

Wet Weather Water Quality Act of 2000

Defining existing uses as those which actually occurred since November 1975 also is inconsistent with EPA’s 1994 Combined Sewer Overflow Control Policy, which was codified into law by Congress under the Wet Weather Water Quality Act of 2000.  Congress required that each permit, order or decree issued pursuant to the Clean Water Act from a municipal combined sewer system “shall conform” to the CSO Control Policy. The Policy includes the following language:

“An existing use is a use actually attained in the water body on or after November 28, 1975. Furthermore, a State may not remove a designated use that will be attained by implementing the technology-based effluent limits required under Sections 301(b) and 306 of the CWA and by implementing cost-effective and reasonable best management practices for nonpoint source controls. Thus, if a State has a reasonable basis to determine that the current designated use could be attained after implementation of the technology-based controls of the CWA, then the use could not be removed.”

This language is significant because it (1) uses “actually attained” in determining what is an “existing use” for purposes of CSO control; and (2) the subsequent sentences make clear that the measure of attainment is the water quality that would result from the technology-based controls– not what occurs, but what is attained.

IDEM’s Guidance on Existing Use

To assist municipalities in making the existing use determination, IDEM provides guidance language and principles that it will apply.  IDEM’s specific guidance and two of the “principles” are inconsistent with the federal legal framework and guidance discussed above, and they are of particular concern because if applied, could lead to an untenable result for many Indiana CSO communities.   IDEM first states:

An existing recreational use can be established by demonstrating that:

- fishing, swimming or other recreational uses have actually occurred on or after November 28, 1975; or

- the water quality is suitable to allow the recreational use to be attained – unless there are physical problems, such as substrate or flow that prevent the use from being attained.

See Guidance Document, p. 50 (emphasis added).   IDEM’s first statement, that an existing recreational use can be established by demonstrating that fishing, swimming or other recreational uses have “actually occurred,” is unsupported by statute, regulation or otherwise.  If the determination were that simple, the regulation would use the term “actually occurred” rather than “actually attained.” Furthermore, the “actually occurred” interpretation of existing use does not make sense in the context of analyzing the six chemical, physical and economic factors allowed for a use attainability analysis. Factors 1-3 in 40 CFR 131.10(g) refer to the following conditions as potentially preventing or precluding “attainment of the use:” 1) naturally occurring pollutant concentrations; 2) natural, ephemeral, intermittent, or low-flow conditions; and/or 3) human-caused conditions or sources of pollution.  None of these factors would prevent or preclude actual contact with a waterway, yet EPA lists them as conditions that might prevent “attainment” of the use. EPA provided those six factors because EPA recognized that attainment of water quality to protect designated uses is not always possible.  Where attainment is not possible, EPA allows the use to be modified through the UAA process.  If attainment of recreational uses was equivalent to actual recreational use, then there would be few, if any, water bodies without an existing use.  All water bodies, particularly in urban areas, would be permanently designated for all recreational uses and ineligible for a use revision – even if the water quality standards are not attainable.  This is contrary to the intent of SEA 431 and EPA policy, guidance and regulations.

IDEM’s guidance goes on to explain the specific principles it will apply in making an existing use determination.  The first principle of concern states:

Indiana is determined to take all reasonable steps to protect all people who recreate in its waters, especially children.  Therefore, there is a presumption of finding in favor of finding an existing use for full-body contact recreation if the water is free of physical hazards and accessible when flowing near residential neighborhoods, parks, or schools.  Indiana recognizes that some of these waters may be too shallow during dry periods of the year to allow for adult swimming activities.  However, Indiana also recognizes that:  (a) children will still splash in these streams and may ingest the water, and (b) wet weather events that trigger CSO discharges often provide additional flow that attracts people, especially children, to the water during such times.

See Guidance Document, p. 51, principle 1 (emphasis added).  The second principle of concern states:

If there is an actual recreational use of the water during or immediately after a significant wet weather event [except for occasional or incidental use by individual adults], then an existing use has been established.

See Guidance Document, p. 51, principle 4 (emphasis added).

The City of Indianapolis supports IDEM’s underlying goal:  to take all reasonable steps to protect people who recreate in Indiana’s waters, especially children.  Federal law provides that actual uses can and should be protected through the protection of sensitive and priority areas for CSO control.  Shellfish beds, beaches and other important uses have been protected in other CSO communities through the sensitive area provisions of EPA’s CSO policy.  Information on actual uses will be important to a community in developing its long-term plan for controlling CSOs, including identifying sensitive areas, selecting appropriate control technologies, prioritizing projects, and taking steps to educate and notify citizens about the potential risks of contact with CSO-impacted waters.

Equating actual use with existing use, however, seems to place CSO communities in the unenviable position of permanent non-compliance with water quality standards if CSOs, stormwater and other pollution sources cannot be controlled to a level that meets the standards.  This is the potential Catch-22 for CSO communities:  If actual recreational uses are interpreted to equate to an existing use, then recreational standards for E. coli bacteria may not be temporarily suspended through the UAA process.  Yet if the standards are not attainable due to chemical, physical or socioeconomic factors, then the community cannot avoid non-compliance with water quality standards.  Therefore, a community could be in a position where the standards have not been attained and are not attainable, yet no regulatory relief is available because people have chosen to recreate in unsafe waters.  In our view, the only existing use that has been “attained” in this scenario is recreation in unsafe waters.

Figure 1 on the following page illustrates the suggested relationship among designated uses, water quality standards, and existing uses, based upon the analysis and conclusions in this discussion paper.

Periodic review to determine whether higher uses can be attained

Figure 1

Suggested Questions for Discussion

      - Is it possible that water quality standards to protect actual recreational uses of a waterbody might not be attainable due to chemical, physical or economic factors? If so, how can we say that the designated use has been “attained” and is an existing use?

      - Should we say swimming is an “existing use” during periods when we know the bacteria standards necessary to protect human health are exceeded? Is that consistent with our responsibility to provide meaningful public notice and information on the safety of swimming in urban waterways?

      - If a water body does not and has not met recreational water quality standards during and immediately following wet weather events, how and why would we say the recreational use has been “attained” during wet weather, even if actual uses have occurred?

      - How should a community use information on actual uses of waterways as it develops its long-term control plan for CSOs?

      - If a use is not attainable due to natural conditions, urbanization or widespread socio-economic impacts, isn’t it the responsibility of the community, IDEM and EPA to curb actual uses through notification, education or other means?

      - Under what circumstances would you support a temporary suspension of the designated use and associated water quality standards following wet-weather events in urban waterways, as allowed under state and federal law?